| Function | Analog 0- 10VDC | CW Relay | CW Relay | Drive Enable | GND | N/C | CCW Relay | CCW Relay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RJ45 Pins | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Colors | White & Orange | Orange | White & Green | Blue | White & Blue | Green | White & Brown | Brown |
| Your Connections | A1 | S1 | SC | ----- | AC | ---- | SC | S2 |

This example table is for a Yaskawa V1000 Inverter. You can see the diagram below of each connection point and compare that to the diagram of your inverter/VFD to make the corresponding connections. In this example the SC connection is tied directly to a switch for sink or source (24V or GND) so you can just connect it and it should work. Some units have 2 connection points and you have to use the one corresponding to the switch setting, but that is not the case with this example.