| Function | Analog 0- 10VDC | CW Relay | CW Relay | Drive Enable | GND | N/C | CCW Relay | CCW Relay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RJ45 Pins | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Colors | White & Orange | Orange | White & Green | Blue | White & Blue | Green | White & Brown | Brown |
| Your Connections | AIN | S1 | COM or 24 | ----- | COM | ---- | COM or 24 | S2 |

This example table is for a TECO / Westinghouse N3 Inverter. You can see the diagram below of each connection point and compare that to the diagram of your inverter/VFD to make the corresponding connections. In this example the COM or 24 connection is separate connection point depending on the setting of the SW1 switch. Use the connection point for the setting your switch is set to.  Some units have 1 connection point and you can just change the switch setting and it uses whichever setting the switch is for that point but that is not the case with this style.